MODULAR ROBOT BRAIN

Functional Master Specification

SO-101 / public multi-embodiment datasets / JEPA world model / flow-matching motor control / 1.024B local system

| Field | Baseline |
| --- | --- |
| Version | 1.3 |
| Date | 28 August 2026 |
| Status | Dataset-first zero-paid-cloud architecture, training/data plan, repository structure and implementation learning guide |
| Primary robot | LeRobot SO-101 single arm |
| Parameter budget | 1,024M neural parameters total |
| Runtime | Local; no external LLM API dependency |
| Paid cloud budget | $0 baseline; public datasets replace custom paid simulation generation |

Purpose: one implementation reference for architecture, data, repository structure, training order, hardware allocation, integration, quantization, evaluation and the terminology/layer knowledge required to understand and eventually reimplement the models.

# Executive Summary

The project is a modular local robot brain for the SO-101 arm. Perception and environment prediction use a shared multimodal JEPA representation; high-level reasoning remains a decoder Transformer; motor control uses a pi-inspired flow-matching action model; precise body prediction is kept separate from the world model. Training is dataset-first: diverse public synthetic and real-robot datasets provide broad embodied pretraining, then public and self-collected SO-101 data specialize the body-specific modules. No paid cloud compute or custom large-scale Isaac data generation is required by the baseline plan.

Core rule: high-level models decide what should happen; the Motor Cortex decides how the body should move; learned Body Dynamics and JEPA World Prediction estimate consequences before execution; hard deterministic safety rules remain outside all neural models.

## Fixed top-level decisions

Eight neural modules totaling 1,024M parameters.

Local Language + Speech model is retained; no GPT/API dependency.

Multimodal JEPA Encoder replaces the previous standalone Sensor Fusion design and shares its latent state with the JEPA World Predictor.

Executive stays primarily an autoregressive decoder Transformer; a small Stage-JEPA head may predict useful next-task-stage latents.

Motor Cortex consumes actuator state + physical motor goal only; it never consumes language or camera pixels.

Body Dynamics is SO-101-specific and predicts precise future actuator state and uncertainty.

Training is staged: code-scale test models -> public synthetic/real dataset ingestion -> broad multi-embodiment pretraining -> SO-101 specialization -> subsystem alignment -> full-agent integration -> own SO-101 teleop adaptation -> optional local quantization.

Repository/data discipline and educational reimplementations are mandatory: immutable raw data, reproducible cleaned/normalized tiers, and tiny readable versions of each neural component in learn_lab/.

Paid cloud compute is outside the baseline scope. Public Isaac/Isaac Lab datasets are consumed as fixed source corpora; the RTX 3050 laptop is used for tiny correctness models and the RTX 3060 OC 12 GB PC is the primary trainer for individual specialists and integration work.

# 1. System Objective and Functional Scope

## 1.1 Target behavior

A user command such as "pick up the red ball" must be converted into a physically grounded task and executed closed-loop. The system must identify the requested object, select a manipulation strategy, generate physical subgoals, test candidate futures, execute only a short safe prefix, observe reality, recover from error and store relevant outcomes.

Human speech/text
  -> Language + Speech -> TASK_GOAL
  -> Executive -> physical subgoal
  -> IK / goal conversion -> MOTOR_GOAL
  -> Motor Cortex -> K candidate action chunks
  -> Body Dynamics + JEPA World Predictor -> predicted futures
  -> fast scorer / Executive critic -> select or revise
  -> execute short prefix -> sense -> replan -> memory update

## 1.2 Non-goals

No raw PWM/voltage generation by neural models; normal servo position control remains underneath the learned controller.

No all-in-one VLA. Language, perception, planning and motor control remain separable and independently trainable.

No pixel-generative world model as the baseline. Environment prediction is primarily in JEPA latent space.

No use of model weights as the primary episodic memory store.

No requirement that slow reasoning complete before every motor tick.

# 2. Final 1.024B Functional Architecture

| Module | Budget | Core architecture | Primary function |
| --- | --- | --- | --- |
| Multimodal JEPA Encoder | 280M | ViT-style visual context encoder + multimodal fusion/resampler | Encode cameras, IMU and proprioception into compact grounded world latents z_t. |
| JEPA World Predictor | 160M | Action-conditioned latent predictor Transformer | Predict future environment/world latents under candidate actions/subgoals. |
| Motor Cortex | 128M | Flow-matching Transformer action expert | Generate K short SO-101 joint-position action chunks from current body state + motor goal. |
| Body Dynamics | 32M | Action-conditioned dynamics Transformer | Predict precise future SO-101 q/qdot/load distribution and uncertainty for each action candidate. |
| Executive Brain | 330M | Decoder Transformer + small Stage-JEPA predictor | Reason, ground tasks, retrieve memory, compare predictions, generate/revise physical subgoals. |
| Language + Speech | 80M | Conformer speech encoder + text/semantic Transformer | Speech/text understanding and structured task semantics; optional local response generation. |
| Memory Controller | 12M | Small Transformer/retrieval controller | Write, retrieve, rank and compress external working/short/long-term memories. |
| Orchestrator | 2M | Small router + deterministic message bus | Route typed requests/results, priorities, timeouts and capability calls. |

Parameter total: 280 + 160 + 128 + 32 + 330 + 80 + 12 + 2 = 1,024M.

## 2.1 Runtime time-scale hierarchy

| Layer | Target rate | Responsibility |
| --- | --- | --- |
| Servo firmware/PID | hardware-native | Electrical/current/position stabilization and hard servo limits. |
| Motor Cortex + Body Dynamics | 30 Hz initial; 30-50 Hz target | Candidate action generation, body prediction, receding-horizon correction. |
| Multimodal JEPA Encoder | 10-30 Hz | Maintain compact fused world representation from sensors. |
| JEPA World Predictor | 5-15 Hz / on demand | Environment consequence prediction for candidate actions or subgoals. |
| Executive | 1-10 Hz / event-driven | Planning, criticism, recovery, subgoal generation. |
| Language, Memory, Orchestrator | event-driven | Human interface, retrieval/write operations and routing. |

# 3. Common Neural Bus and Typed Interfaces

All modules expose adapters to a common 512-dimensional neural bus. Numeric physical fields remain typed numeric tensors; they are not converted to English. Latent observations are transported as N x 512 tokens. The orchestrator validates and routes packets but does not make task decisions.

Header: message_type, source, destination, timestamp, sequence_id, priority, confidence, frame_id, validity_mask
Payload: typed numeric tensors and/or N x 512 latent tokens

| Message | Producer -> consumer | Required payload |
| --- | --- | --- |
| TASK_GOAL | Language -> Executive | intent, entities, attributes, constraints, optional language tokens |
| WORLD_STATE | JEPA Encoder -> Executive/World/Memory | world tokens z_t + structured robot/object state where available |
| MOTOR_GOAL | Executive/IK -> Motor Cortex | q_goal[6], qdot_goal[6], duration, precision/speed constraints |
| ACTION_CANDIDATES | Motor Cortex -> Body/World | K x H x 6 relative joint-position commands |
| BODY_PREDICTION | Body -> scorer/Executive | future q/qdot/load mean + uncertainty for each candidate |
| WORLD_PREDICTION | World -> Executive/scorer | future latent, object/contact/collision/grasp event probabilities |
| MEMORY_QUERY/RESULT | Executive <-> Memory | query tokens/filters; ranked evidence with timestamps/confidence |
| EXECUTION_RESULT | Robot stack -> Executive/Memory | actual state, prediction residual, success/failure/event flags |

# 4. Individual Model Functional Specifications

## 4.1 Multimodal JEPA Encoder - 280M

Purpose: learn a compact semantic and physical representation of the current world from cameras, IMU and actuator telemetry. It is the primary place where concepts such as object identity, color, spatial relations, motion and contact cues become grounded.

| Item | Baseline |
| --- | --- |
| Visual context encoder | ViT-style shared camera encoder; target approximately 18-20 blocks at d_model about 896. Same weights for all camera views. |
| Fusion/resampler | Sensor type + timestamp embeddings; 4-6 multimodal fusion blocks; resample to about 32 compact world tokens. |
| Inputs | 1-2 RGB cameras initially; q/qdot/previous command; IMU; optional motor current/load/temperature. |
| Training objective | JEPA masked/context prediction in latent space; temporal prediction for video; optional supervised object/attribute/pose heads. |
| Output | z_t world tokens + optional structured object table with pose/class/attributes/confidence. |
| Key constraint | Do not require raw-pixel reconstruction. Preserve geometry and task-relevant information rather than texture detail. |

I-JEPA shows semantic representation learning by predicting target embeddings rather than pixels [6]. V-JEPA 2 extends the approach to video/world prediction and action-conditioned robot planning [7][8].

## 4.2 JEPA World Predictor - 160M

| Item | Baseline |
| --- | --- |
| Core | Approximately 20-22 Transformer blocks, d_model about 768, GQA, SwiGLU, RMSNorm/RoPE. |
| Inputs | Current JEPA world latent z_t + candidate action/subgoal + optional Body Dynamics prediction + short-term memory context. |
| Outputs | Predicted future world latents z_hat[t+1:t+H], object/contact/grasp/collision event heads and uncertainty. |
| Training | Action-free temporal prediction first, then action-conditioned robot trajectories, then SO-101 specialization. |
| Use | Executive compares predicted future states/events against task goals and safety constraints. |
| Non-use | Does not replace precise SO-101 joint dynamics; Body Dynamics remains separate. |

## 4.3 Motor Cortex - 128M

| Item | Baseline |
| --- | --- |
| Core | 20 Transformer blocks, d_model=768, 12 query heads, 4 KV heads, SwiGLU FFN about 2048, RMSNorm, RoPE. |
| Flow conditioning | AdaRMSNorm/equivalent flow-time conditioning. |
| Current-state input | q[6], qdot[6], previous command[6]; add motor current/load/temperature only if reliable. |
| Goal input | q_goal[6], qdot_goal[6], duration + optional precision/speed constraints. |
| Action representation | Relative joint-position chunks; baseline horizon H=16. |
| Inference | Sample K=4 candidates initially with about 8-10 flow steps; tune for latency. |
| Output | K x H x 6 candidate action chunks + policy confidence. |

The design adapts the specialized action-expert and flow-matching ideas used by pi/openpi, while removing vision and language from the low-level motor model [1][2][3].

## 4.4 Body Dynamics - 32M

| Item | Baseline |
| --- | --- |
| Core | 10 Transformer blocks, d_model=512, 8 query heads, 2 KV heads, SwiGLU FFN about 1536, RMSNorm. |
| Inputs | Current SO-101 actuator state + one H x 6 candidate action chunk, batched across K candidates. |
| Outputs | Per future step: mean and calibrated uncertainty for q[6], qdot[6], optional load/current/temperature. |
| Training | Public SO-101 synthetic trajectories -> public real SO-101 logs -> own hardware adaptation. Real data receives increasing weight as it becomes available. |
| Scope | Embodiment-specific. Cross-robot data is not used as direct target dynamics unless explicitly conditioned on embodiment. |

## 4.5 Executive Brain - 330M

| Item | Baseline |
| --- | --- |
| Main core | Approximately 24 decoder-Transformer layers, d_model=1024, 16 query heads, 4 KV heads, SwiGLU FFN about 2816, RMSNorm/RoPE. |
| Stage-JEPA | Small approximately 20-24M latent predictor inside the 330M budget; predicts useful next task-stage latent/subgoal representation. |
| Inputs | TASK_GOAL, WORLD_STATE, working memory, retrieved memory, BODY_PREDICTION, WORLD_PREDICTION, task history. |
| Outputs | Structured physical subgoal, constraints, recovery action, request for more sensing/memory/prediction, or task completion. |
| Role | Central planner/critic; does not generate servo commands. |
| Training | Multi-robot task graphs/demonstrations, public synthetic and real successes/failures, prediction-aware choice tasks, then SO-101 recovery/fine-tuning. |

JEPA is used as an auxiliary predictive representation for subgoal/task-stage forecasting; it does not replace the decoder because the Executive must perform variable-length structured reasoning and tool/module selection.

## 4.6 Language + Speech - 80M

| Item | Baseline |
| --- | --- |
| Speech encoder | Conformer, about 8 layers, d_model about 384, 6 heads, convolution modules. |
| Semantic/text core | About 10 Transformer layers, d_model about 512, 8 query heads, GQA, SwiGLU/RMSNorm/RoPE. |
| Inputs | Microphone audio and/or typed text. |
| Outputs | Structured TASK_GOAL: intent, entity, attributes, relations, constraints; optional local text response. |
| Training | Broad speech/text pretraining + robot-domain instructions + synthetic paraphrases + noisy-room augmentation. |
| Boundary | Deep physical planning stays in Executive. |

## 4.7 Memory Controller - 12M + external stores

| Store | Retention | Implementation / content |
| --- | --- | --- |
| Working | seconds | In-RAM blackboard: current goal, target object, latest subgoal, prediction error, gripper state. |
| Short-term | seconds to minutes | Ring buffer of compressed JEPA world tokens, actions, failures and selected frames/events. |
| Long-term | hours to months | Vector + structured episodic database: object locations, task outcomes, procedures, repeated failures, semantic summaries. |

The 12M controller ranks reads/writes, compresses old episodes and decides when evidence is stale. Memory data is not counted in the 1.024B neural parameter budget. Multi-scale embodied memory research supports separating dense recent context from compressed long-term memory [12].

## 4.8 Orchestrator - 2M

The orchestrator is mostly deterministic software plus a small learned router. It validates schemas, routes messages, maintains priorities/timeouts/correlation IDs, rejects stale data and serves module capability requests. It does not make task plans and must not become a second Executive.

# 5. Semantic Grounding and Example Task

The command "pick up the red ball" requires three separately learned mappings: language meaning, visual grounding and manipulation skill. Motor control alone cannot learn these semantics.

| Stage | Functional output |
| --- | --- |
| Language | TASK_GOAL = {intent:PICK_UP, class:BALL, color:RED}. |
| JEPA Encoder / supervised grounding heads | Detect/represent candidate scene objects and their attributes/poses. |
| Executive | Bind requested BALL+RED to one observed object; select target and plan grasp stages. |
| Stage-JEPA / World Predictor | Estimate whether candidate subgoal produces a useful next-stage state, e.g. gripper aligned or object lifted. |
| IK / goal converter | Convert end-effector pose to reachable SO-101 joint target. |
| Motor Cortex | Generate candidate SO-101 action chunks. |
| Body + World prediction | Estimate actual actuator response and environment consequence. |
| Execution | Select, execute short prefix, observe, replan and verify grasp. |

# 6. Closed-Loop Motor and Prediction Control

At each motor cycle:
1. Read s_t and current MOTOR_GOAL.
2. Motor Cortex samples U_1...U_K.
3. Body Dynamics predicts precise body futures for all candidates.
4. JEPA World Predictor is invoked when environment interaction matters.
5. Deterministic scorer / Executive critic ranks candidates.
6. Execute only the first 2-4 commands of the selected chunk.
7. Read actual state; compute prediction residual; replan.

## 6.1 Fast deterministic MPC score

J(U) = w_goal * final_goal_error
     + w_vel * final_velocity_error
     + w_smooth * jerk/smoothness
     + w_limit * joint/workspace penalty
     + w_unc * body/world uncertainty
     + w_collision * predicted collision risk
     + w_task * task-specific event cost

Prediction must influence action selection. A parallel future-state output head beside the policy is insufficient because the action cannot use the prediction generated in the same pass. Candidate generation -> prediction -> scoring -> short-prefix execution provides the required "imagine then move" loop and is consistent with learned model-predictive control methods such as TD-MPC2 and Dreamer [4][5].

# 7. Training Data Architecture

## 7.1 Canonical episode schema

episode_metadata: source, license, robot_id, embodiment, task, success/failure, quality
observation_t: RGB views, q, qdot, prev_cmd, IMU, optional current/load/temp, timestamp
action_t: native robot action + normalized end-effector action where derivable
observation_t+1: next sensors/state
language: instruction / task label / paraphrases
scene_metadata: objects, attributes, 3D pose, contacts, segmentation when available

All public datasets are converted to this canonical schema. Native actions are preserved. Cross-robot action transfer uses a normalized gripper-frame representation such as delta XYZ + delta roll/pitch/yaw + gripper, following the standardization used by Open X-Embodiment [9].

## 7.2 Data source priorities

| Source group | Domain | Primary modules | Project rule |
| --- | --- | --- | --- |
| Public SO-101 Isaac/LeIsaac sets | Synthetic; direct embodiment | Motor, Body, JEPA, World, Executive | Highest-value synthetic source for SO-101 geometry/action semantics; tag every episode sim. |
| Public Franka/Isaac sets | Synthetic; other embodiment | JEPA, World, Executive | Use for visual/contact/task diversity; convert actions to universal task-space where valid. |
| Piper / UR5e simulation sets | Synthetic; other embodiment | JEPA, World, Executive | Use for harvesting, occlusion, insertion and force/contact diversity; never treat native joints as SO-101 targets. |
| ArmnetBench SO-101 | Real; direct embodiment | Motor, Body, JEPA, World, Executive | Use successful teleop/rollouts for imitation; failures/suboptimal for prediction, critic and recovery. |
| Project IRA SO-101 | Real; direct embodiment | Motor, Body, JEPA, Executive, language grounding | Strong prompt diversity; single-environment bias means cap its sampling weight. |
| Open X-Embodiment | Real; 22 embodiments | JEPA, World, Executive, generic action representation | Use embodiment tokens + normalized gripper-frame actions; not direct SO-101 dynamics targets. |
| DROID | Real; Franka | JEPA, World, Executive, generic action representation | High scene/task diversity; preserve camera calibration and language annotations. |
| BridgeData V2 | Real; WidowX 250 | JEPA, World, Executive, generic action representation | Use camera-availability masks; balance by task/environment rather than raw trajectory count. |
| Own SO-101 teleop/autonomous | Real; exact target embodiment | Motor, Body, JEPA, World, Executive | Highest final-domain weight; captures exact servos, cameras, workspace, latency and calibration. |

## 7.3 Dataset routing by module

| Module | Public SO-101 sim | Public SO-101 real | Other-robot sim | Other-robot real |
| --- | --- | --- | --- | --- |
| JEPA Encoder | Very high | Very high | Very high | Very high |
| JEPA World Predictor | Very high | Very high | Very high after action normalization | Very high after action normalization |
| Motor Cortex | Very high | Very high | Low: universal-motion pretraining only | Low/medium: universal-motion pretraining only |
| Body Dynamics | High initially | Very high | None as direct targets | None as direct targets |
| Executive | High | High | Very high | Very high |
| Language + Speech | Task text only | Task text + prompts | Task text only | Task text + language annotations |
| Memory Controller | Episode traces | Episode traces | Episode traces | Episode traces |
| Orchestrator | No direct robot-data need | Integration traces | No direct robot-data need | Integration traces |

## 7.4 Public Dataset Registry: Simulation vs Real

The project maintains a pinned dataset registry. Domain labels below are part of the training sample and are never inferred from filename alone. The attached dataset inventory is the baseline list; current source cards must be pinned by revision before bulk download because community datasets can change after publication.

| Dataset | Domain / robot | Use in this project | Priority |
| --- | --- | --- | --- |
| SO101 11-task 3300 | Isaac/LeIsaac simulation; SO-101 | Direct-embodiment visual/action pretraining; task diversity; Motor/Body/JEPA/World/Executive. | A |
| SO101 MA MultiTask 700 | Isaac Lab simulation; SO-101 | 7-task direct-embodiment multi-task training; dual-camera JEPA; Motor/Body specialization. | A |
| SO101 Pick Orange + Fold Towel + Sort/Stack/Lever/Box/Extract sets | Isaac/LeIsaac simulation; SO-101 | Fill skill gaps; color grounding, deformable cloth, box/lever manipulation, grasp/place. | A |
| Franka Place Cube Into Box - Isaac Lab Mimic | Isaac Lab simulation; Franka | Dual-camera visuomotor, object/subtask labels, generic pick/place world prediction. | B |
| MT-LIBERO Isaac Lab package | Human LIBERO demos repackaged into Isaac Lab state layout; Franka | 40 tasks; state-only task/world/executive training. Treat provenance as MuJoCo demonstrations, not Isaac re-simulation. | B |
| NVIDIA PhysicalAI Manipulation SingleArm | Isaac Sim synthetic; Franka | Stack/cabinet/drawer visual and world-dynamics diversity. | B |
| NVIDIA PhysicalAI Manipulation Augmented | Isaac Lab Mimic + Cosmos visual augmentation; Franka | RGB/depth/segmentation/normals for JEPA auxiliary pretraining and sim-to-real visual robustness. | B |
| DynaSoftGrasp M1 Soft Mouse | Isaac Sim soft-body simulation; Franka | Deformable grasp/world uncertainty; 3-camera visual prediction. | B |
| SoftVTBench | Isaac Sim; Franka + tactile | Optional deformable/tactile representation and safety prediction; visual/proprio subset remains usable without tactile hardware. | C |
| AssembleBench | Isaac Lab synthetic scripted expert; Franka + Robotiq | Contact-rich peg/gear/nut assembly; Executive/World; generic action representation only. | B |
| Assembly Joint / Pick Insert | Isaac Lab synthetic; Franka + Robotiq | Precision insertion, wrench/contact cues, generic action/world prediction. | B |
| Piper Apple Picking 1M | Isaac Lab/Isaac Sim; AgileX Piper | 1M-frame harvesting/occlusion dataset; bounding boxes are auxiliary JEPA targets. | B |
| UR5e SFP Connector Insertion v5 | Simulation challenge data; UR5e | Precision insertion, multi-camera and force/contact-oriented task structure. | C |
| ArmnetBench SO-101 | Real SO-101 arm farm; teleop + policy rollouts | Direct real embodiment, success/failure/suboptimal labels; final SO-101 specialization and critic/world learning. | A |
| Project IRA SO-101 930 | Real SO-101 teleoperation | 930 episodes / 93 prompt variants; language grounding + direct embodiment policy/body adaptation. | A |
| Open X-Embodiment | Real; 22 robot embodiments | Large-scale cross-embodiment perception, task semantics and universal gripper-frame action pretraining. | B |
| DROID | Real Franka; 76k demonstrations | Scene/task diversity, calibrated cameras, language; JEPA/World/Executive/general action representation. | B |
| BridgeData V2 | Real WidowX 250; 60,096 trajectories | 24 environments, 13 skills, language; visual/general manipulation pretraining. | B |
| Own SO-101 teleop + autonomous logs | Real target hardware | Final domain adaptation, body residuals, recovery and personalized workspace competence. | A+ |

Priority A is downloaded/validated first. B is added after loaders and sampling are stable. C is optional or task-specific. Very large datasets should be subset-tested before full download.

## 7.5 Canonical Cleaning Pipeline

| Step | Required operation |
| --- | --- |
| 1. Register | Record dataset_id, exact revision/commit, source URL, license, domain=sim/real, robot_id, FPS, camera names, native action semantics and known unit conventions. |
| 2. Integrity | Verify files/checksums, episode lengths, monotonic timestamps, video decodability, frame-to-state alignment, NaN/Inf, missing columns and duplicate episodes. |
| 3. Canonicalize | Map fields into canonical names without destroying source values. Preserve native state/action arrays and original timestamps. |
| 4. Units/frames | Convert to SI/radians only when source metadata proves the unit/frame. Store source and canonical coordinate-frame metadata. Never guess from numeric range. |
| 5. Time | Choose model-specific sample rate; resample continuous state carefully, never interpolate across episode boundaries/discontinuities; create validity masks. |
| 6. Derived signals | Derive qdot only when absent using timestamp-aware finite differences + smoothing/clipping; derive previous command, action chunks, future states and task-stage boundaries. |
| 7. Quality labels | Keep success/failure/suboptimal and source-policy identity. Never silently drop failures; route them differently by model. |
| 8. Split | Split by episode and, when possible, scene/task/source session. Keep near-duplicate rollouts and frames from the same episode in one split. |
| 9. Normalize | Compute training-only statistics per modality/action representation; save statistics with the dataset manifest and checkpoint. |
| 10. Shard | Create immutable model-specific shard manifests so each model receives only fields it is allowed to use. |

## 7.6 Model-Specific Dataset Selection and Cleanup

| Model | Datasets to use | Model-specific cleanup / fields kept | Training shard |
| --- | --- | --- | --- |
| JEPA Encoder | All public sim with RGB; ArmnetBench; Project IRA; OXE; DROID; BridgeData; own SO-101. MT-LIBERO only if visual data is added elsewhere. | Decode/align cameras; resize/crop consistently; normalize RGB; camera/modality availability masks; retain proprio/IMU when present; balance by dataset/task not frame count. Segmentation/depth/bboxes/tactile are auxiliary targets only. Remove corrupted video/unaligned frames, not difficult behaviors. | shards/jepa_encoder/{sim,real}/: context frames, target frames, sensor tokens, masks, domain + embodiment ids, optional auxiliary labels. |
| JEPA World Predictor | All sequential sim/real datasets with actions; strongest: NVIDIA SingleArm/Augmented, Franka Mimic, DynaSoftGrasp, Assemble/Assembly, SO-101 sets, OXE/DROID/Bridge, own SO-101. | Require valid t -> action -> t+k alignment. Convert actions to canonical EE delta only when kinematics/frames are known; keep embodiment token. Keep failures and contacts. Privileged sim object poses/contact labels may supervise event heads but are excluded from runtime inputs. | shards/world/: z-source episode refs, canonical action, future targets, event labels, domain/embodiment, validity/uncertainty masks. |
| Motor Cortex | Public SO-101 sim sets + successful real SO-101 teleop/ArmnetBench/Project IRA + own successful/recovery demonstrations. Other robots only for optional universal EE-action pretraining. | Drop camera/audio for core motor training. Keep q, derived qdot, previous command, goal/future target, native SO-101 joint action. Reject corrupt/unsafe action spikes; do not imitate labelled failures. Build H-step chunks and hindsight goals from later states. Normalize per SO-101 joint. | shards/motor/: state_t, motor_goal, action_chunk[H,6], constraints, source/domain/quality weight. |
| Body Dynamics | Public SO-101 sim + all valid real SO-101 trajectories (including failed task attempts) + own hardware logs. | Keep actuator state, action, future actuator state. Task success is irrelevant unless sensor/action data is corrupt. Derive qdot consistently; preserve command latency/timestamps; label sim vs real; estimate per-source noise. Final training upweights own real robot. | shards/body/: s_t, action_chunk, future q/qdot/(load if available), timestep, domain, robot/session id. |
| Executive Brain | All task-labelled sim/real datasets; especially MT-LIBERO, NVIDIA task sets, SO-101 multi-task, ArmnetBench labels, Project IRA prompts, DROID/Bridge/OXE language. | Canonicalize task text/entities; create task-stage/subgoal sequences from annotations or trajectory segmentation. Use success/failure as positive/negative planning traces. Privileged sim state may generate target subgoals but must not appear in runtime-equivalent input fields. Balance tasks/skills rather than frames. | shards/executive/: task_goal, world-summary refs, memory context, candidate outcomes, chosen subgoal, success/recovery labels. |
| Language + Speech | Robot instruction text from Project IRA, DROID, Bridge, OXE, SO-101/Isaac task labels; separate general public speech corpus is required for acoustic pretraining. | Unicode/text normalization, remove duplicated boilerplate, preserve synonyms/paraphrases, canonical intent/entity/attribute labels, train/val split by semantic template to test paraphrase generalization. Robot datasets provide text grounding, not sufficient speech audio. | shards/language/: text/audio refs, intent/entity/attribute targets, robot-task paraphrase groups. |
| Memory Controller | Long episodes/traces from all datasets + integrated robot logs. | Convert trajectories into event summaries; create write/ignore/compress labels, retrieval query-positive-negative triples, temporal ordering and stale-memory examples. Keep dataset/task/session provenance so retrieval cannot leak future frames. | shards/memory/: event stream, memory operations, queries, positives/negatives, timestamps, retention class. |
| Orchestrator | No direct imitation dataset requirement; derive typed message traces from all module schemas and integration tests. | Generate synthetic routing/load/staleness/failure cases. Remove robot-specific content from routing target; train only source/destination/type/priority/timeout/correlation behavior. | shards/orchestrator/: message header/payload shape metadata, route, priority, timeout and expected validation result. |

### 7.6.1 Sampling and Weighting Rules

Sample by dataset/task/domain buckets, not uniformly by raw frames. Piper 1M or OXE must not drown out smaller direct SO-101 sets.

For JEPA/World/Executive, preserve substantial other-robot diversity throughout training. For Motor/Body, progressively increase direct SO-101 weight.

Separate imitation weight from prediction weight: a failed trajectory can have imitation_weight=0 but world/body/critic_weight=1.

Keep source_policy and collection_method (teleop, scripted expert, learned-policy rollout, synthetic Mimic) as metadata; quality differs even within one dataset.

Use domain-balanced validation: held-out synthetic source, held-out real non-SO-101 source, held-out public real SO-101, and later own-hardware holdout.

## 7.7 Public synthetic-data handling

| Rule | Treatment |
| --- | --- |
| Domain tag | Every synthetic episode stores domain=sim, simulator family/version if known, robot embodiment and source dataset revision. |
| Privileged state | Object poses, segmentation, contacts and physics state may be used as training targets/auxiliary labels; they are not runtime inputs unless the physical robot can observe them. |
| Visual diversity | Prefer already randomized/Cosmos-augmented views; add only cheap local image augmentation that preserves geometry and task identity. |
| Action semantics | Keep native action unchanged in raw/cleaned tiers; create a separate universal EE-space action only when coordinate frames and units are known. |
| Mixing | Do not let large synthetic sets dominate real data by frame count; sample by dataset/task/domain with caps. |
| Dynamics use | Other-robot synthetic dynamics never supervise SO-101 Body Dynamics directly. Public SO-101 synthetic dynamics are useful pretraining but are later corrected by real SO-101 data. |

Public Isaac/Isaac Lab datasets are used as fixed training corpora; the baseline does not require generating new large-scale simulation data. Isaac replay tools may still be used locally for schema inspection, trajectory replay, or future zero-cost experiments. The NVIDIA SO-101 Isaac workflow remains a useful implementation reference [15].

## 7.8 Data quality, leakage and licensing

Do not imitate failed trajectories with the Motor Cortex. Use failures as World/Body prediction, critic, uncertainty and recovery data.

Maintain per-episode source, robot embodiment, success, quality and domain weights. Do not treat one million weak episodes as equivalent to high-quality teleop.

Keep a dataset manifest with source URL, version, license, citation, redistribution terms and derivative requirements before any merged training release.

Retain a balanced replay mixture of public synthetic and public real multi-robot data during own-hardware adaptation to reduce catastrophic forgetting.

# 8. End-to-End Training Curriculum

## 8.1 Stage 0 - laptop test models: code correctness only

Before any full-size training or bulk dataset download, build miniature versions of every module on the RTX 3050 laptop with 4 GB VRAM. These models are not performance baselines. Their purpose is to prove the software stack, tensor schemas, losses, loaders, checkpoints and closed-loop logic.

| Test model | Suggested tiny scale | Pass condition |
| --- | --- | --- |
| JEPA Encoder + Predictor | 4-6 blocks, d=192-256, low-resolution 64-128 px inputs; roughly 8-15M combined | Loss decreases; masked/future latent prediction works; sensor dropout masks work. |
| Motor Cortex | 4 blocks, d=256; roughly 4-6M | Can overfit a tiny synthetic trajectory set; flow sampling returns valid H x 6 actions. |
| Body Dynamics | 3-4 blocks, d=192-256; roughly 2-4M | Overfits small deterministic dynamics set and uncertainty head remains finite. |
| Executive | 6 blocks, d=256, small 4K vocab; roughly 8-12M | Maps toy TASK_GOAL + WORLD_STATE to correct structured subgoal. |
| Language/Speech | small Conformer + 4-6 text layers; roughly 5-10M | Toy ASR/intent path works end-to-end. |
| Memory/Orchestrator | sub-million to 2M | Read/write/routing schemas, stale-data handling and request correlation pass. |

Laptop test suite must verify: forward/backward pass, loss finite, tiny-set overfit, save/load, deterministic seed, mixed-precision fallback, bus serialization, integration mocks and a toy closed-loop episode.

Do not run full-size model training on the 4 GB laptop. Use it for tiny datasets, loader checks, unit tests and educational implementations.

## 8.2 Stage 1 - public dataset acquisition, validation and canonicalization

Download datasets in A -> B -> C priority order. Start with metadata and small subsets, make each adapter pass integrity tests, then download the full source only when its model value and storage cost are justified.

Create a dataset registry that pins exact source revisions and classifies each source as synthetic/real, robot embodiment, task families, modalities, action semantics and license.

Build adapters for LeRobot/Parquet+video, HDF5, RLDS/TFRecord and source-specific layouts. Do not convert formats merely for consistency if a direct loader is reliable.

Keep raw downloads immutable. Generate cleaned, normalized and model-specific shards from code + manifest only; every transformation is reproducible.

Validate direct SO-101 datasets first for Motor/Body, then add visual/task diversity datasets for JEPA/World/Executive, then large cross-embodiment sources.

No paid cloud compute is required. Storage bandwidth and preprocessing time are the main Stage 1 constraints.

## 8.3 Stage 2 - RTX 3060 12 GB: broad individual-model pretraining

The RTX 3060 OC 12 GB is the primary local trainer. Train one specialist at a time. JEPA Encoder, World Predictor and Executive start with a domain-balanced mixture of public synthetic and real multi-robot data; Motor Cortex and Body Dynamics start with direct SO-101 data and only limited embodiment-neutral pretraining from other robots.

| Round | Data mix | What is trained |
| --- | --- | --- |
| 2A: broad representation pretrain | Domain-balanced public sim + public real across robots | JEPA Encoder, World Predictor, Executive; optional generic EE-action representation. |
| 2B: direct SO-101 specialization | Public SO-101 simulation + public real SO-101; diverse replay retained | Motor Cortex, Body Dynamics, SO-101 adapters; JEPA/World/Executive receive direct-embodiment specialization. |
| 2C: pair alignment | Frozen/partially frozen specialist checkpoints + canonical offline episodes | Motor<->Body; JEPA Encoder<->World Predictor; Language<->Executive; Memory<->Executive. |
| 2D: own-hardware adaptation | Own SO-101 teleop/autonomous logs + public replay | Body residuals, Motor correction, camera/domain adaptation, Executive recovery. |

Memory-saving baseline on 12 GB: microbatch 1, gradient accumulation, activation checkpointing, mixed FP16/BF16 only where the card/framework supports it, FlashAttention/memory-efficient attention when compatible, 8-bit optimizer or optimizer/CPU offload where necessary, and progressive unfreezing. The 280-330M modules may be compute-limited even if made to fit in VRAM; throughput must be benchmarked before committing to long dense pretraining runs.

Engineering rule: the RTX 3060 is the default full-size specialist trainer. If a dense 280-330M run is too slow, reduce sequence/image size for early experiments, freeze stable backbones, train adapters/heads first, and use progressive unfreezing. Do not introduce paid cloud compute as a hidden dependency.

## 8.4 Stage 3 - SO-101 specialization and subsystem alignment

Increase public SO-101 real + synthetic sampling for Motor Cortex, Body Dynamics and the SO-101 adapters while retaining diverse replay for JEPA/World/Executive.

Train pairwise interfaces: Motor <-> Body; JEPA Encoder <-> World Predictor; Language <-> Executive; Memory <-> Executive. Use complete public episodes and offline replay rather than requiring a simulator.

Verify offline grounding -> subgoal -> candidate actions -> predictions -> scoring/recovery on held-out episodes. Use replay tools where available, but no new simulation generation is required.

Only selectively unfreeze modules that are proven bottlenecks. Do not train all 1.024B parameters from random initialization.

## 8.5 Stage 4 - full-agent assembly and real SO-101 teleoperation

Connect specialists through the 512D bus with most weights frozen, validate deterministic safety, then collect own SO-101 teleoperation. This is the highest-value domain data because it captures the exact arm, cameras, calibration, workspace, object set, servo response, backlash and latency.

| Priority | Module | Own-hardware objective |
| --- | --- | --- |
| 1 | Body Dynamics | Calibrate exact actuator response, command latency and uncertainty to this physical SO-101; real logs become highest-weight targets. |
| 2 | Motor Cortex | Correct public-data trajectory style to actual servo settling, overshoot, speed and reachable motion. |
| 3 | JEPA Encoder | Adapt to exact cameras, lighting, backgrounds, occlusion and sensor noise. |
| 4 | JEPA World Predictor | Adapt contact/object consequence prediction to the local workspace and object set. |
| 5 | Executive | Learn recovery patterns and planning errors observed on hardware. |
| 6 | Language/Memory/Orchestrator | Fine-tune only if real failures indicate a need. |

Collect successful demonstrations, deliberate recovery trajectories, partial failures, different speeds, starts, object positions, payloads and lighting conditions.

Retain public synthetic and real replay during own-hardware fine-tuning so the system does not collapse to one room, object set or operator style.

## 8.6 Stage 5 - continuous improvement and local deployment optimization

After the first safe integrated hardware policy works, collect autonomous successes, failures and recovery episodes. Promote data into the same raw -> cleaned -> normalized pipeline and selectively fine-tune only the modules that failed.

Run full-system integration/regression tests on the RTX 3060 with frozen specialists loaded sequentially or with memory-conscious execution.

Keep FP32/FP16 reference metrics before any deployment quantization.

Use FP16 as the default local inference/training precision where stable. Optional INT8 or weight-only 4-bit quantization may be evaluated later if supported by the deployment stack and accuracy gates.

Benchmark task success, motor error, body/world prediction calibration, memory footprint and latency against the higher-precision reference.

Export deployment checkpoints, normalization statistics, dataset manifests and any quantization scales/metadata as versioned artifacts.

NVFP4 is not part of the zero-cost baseline because native NVFP4 execution requires Blackwell-class hardware. It remains an optional future export path if suitable hardware becomes available without changing the model interfaces [16][17].

# 9. Compute, Hardware and Storage Plan

| Platform | Role | Allowed work | Not intended for |
| --- | --- | --- | --- |
| Laptop RTX 3050 4 GB | Stage 0 engineering/learning | Tiny models, loaders, unit/integration tests, schemas, losses, checkpointing, small dataset subsets | Full-size specialist training |
| RTX 3060 OC 12 GB PC | Primary training + integration | Train individual specialists sequentially; public-data pretraining; pairwise alignment; own-data fine-tuning; offline integrated inference | Training all 1.024B parameters jointly from scratch; Blackwell-only precision formats |
| CPU + local/external storage | Data engineering | Download, checksum, decode/transcode when required, HDF5/Parquet indexing, manifest generation, sharding and archival | Neural-network throughput work |

## 9.1 No-paid-cloud baseline

| Item | Baseline |
| --- | --- |
| Paid GPU/cloud compute | $0 |
| Mandatory training hardware | Existing RTX 3050 4 GB + RTX 3060 OC 12 GB |
| Primary external cost driver | Storage / internet bandwidth only if additional capacity is needed; not assumed as a mandatory project cost |
| Cloud acceleration | Optional future convenience only; never a dependency of the reproducible baseline |

Cost rule: mandatory paid cloud compute is $0. The project must remain reproducible on the existing RTX 3050 + RTX 3060 hardware; large public datasets are controlled through subset-first downloads, caching, and deterministic manifests.

## 9.2 Bulk-download and training checklist

| Before bulk download | Before full-size training |
| --- | --- |
| Read card + license; pin revision; inspect schema; download a small subset first. | Tiny model overfits a small shard; loader shape/unit/action tests pass. |
| Estimate disk footprint and model value; reject redundant/low-quality sources before full download. | Train/validation/test manifests are frozen; no episode/scene leakage. |
| Create raw manifest with checksums, domain/embodiment/action semantics and provenance. | Sampling weights are set by dataset/task/domain; normalization stats use training split only. |
| Run decode/alignment test and verify timestamp/video coverage. | Checkpoint path, resume behavior, metrics and storage cleanup are tested before long runs. |

# 10. Precision and Local Deployment Compression

| Stage | Precision policy |
| --- | --- |
| Debug / RTX 3050 | FP32 first for correctness; FP16 only after finite-gradient and tiny-overfit tests pass. |
| RTX 3060 training | FP16 mixed precision by default where stable; use BF16 only if the installed runtime/GPU path reports reliable support; keep sensitive reductions/losses in FP32. |
| Memory constrained training | Microbatch 1, gradient accumulation, activation checkpointing, memory-efficient attention, optional 8-bit optimizer/CPU offload. |
| Reference checkpoint | Keep a reproducible FP16/FP32 reference metric set before quantization. |
| Optional local quantization | Evaluate INT8 or weight-only 4-bit only if deployment libraries support the layer and physical accuracy gates pass. |
| NVFP4 | Deferred: not required because the baseline has no Blackwell hardware. Keep interfaces quantization-agnostic for future export. |

1.024B raw parameter weight storage (ignoring scales/metadata):
FP32 ~ 4.096 GB
BF16/FP16 ~ 2.048 GB
INT8/FP8-like 8-bit storage ~ 1.024 GB
4-bit weight-only storage ~ 0.512 GB
Actual deployment size is higher because scales, metadata and higher-precision exception layers are retained.

# 11. Evaluation and Acceptance Gates

| Module | Required measurements |
| --- | --- |
| JEPA Encoder | object/attribute grounding, pose error, temporal consistency, sensor-drop robustness, real-vs-sim embedding stability |
| JEPA World Predictor | future-latent consistency, object/event prediction, collision/grasp calibration, horizon degradation |
| Motor Cortex | goal success, final joint error, overshoot, jerk, action-limit violations, candidate diversity, inference latency |
| Body Dynamics | multi-step q/qdot error, NLL, uncertainty calibration, sim-to-real residual error |
| Executive | subgoal validity, task success, recovery rate, prediction-aware choice quality, unnecessary replans |
| Language + Speech | WER, intent accuracy, entity/attribute slot accuracy, background-noise robustness |
| Memory | retrieval precision/recall, stale-memory rate, long-horizon benefit |
| Orchestrator | routing accuracy, stale packet rejection, queue latency, dropped/duplicated request rate |
| Full system | task success, collisions, hard-limit events, time-to-completion, recovery, control deadline misses, quantized-vs-reference gap |

## 11.1 Mandatory progression gates

Stage 0 gate: every tiny module can overfit a tiny dataset and the complete mocked bus loop runs without schema or numerical failures.

Public-data gate: every dataset adapter passes integrity/unit/action-semantics checks and the target model improves on held-out source/domain splits without leakage.

Public-data gate: JEPA/world/executive improve on held-out real robot/video data without unacceptable SO-101 forgetting.

Hardware gate: real SO-101 can execute constrained reach/grasp primitives with hard safety layer enabled before open-ended task fine-tuning.

Quantization gate: any locally quantized module remains within project-defined tolerance of FP16/FP32 reference metrics. Failed modules remain higher precision.

# 12. Deterministic Safety Layer

Learned predictions are advisory and cannot be the final safety authority. Hard constraints must wrap every neural command.

Joint position, velocity and acceleration clamps.

URDF-based workspace/self-collision constraints and forbidden regions.

Emergency stop independent of the neural runtime.

Command timeout, stale-state rejection and watchdog hold/stop behavior.

Maximum gripper current/force policy based on available telemetry.

Stop/hold when sensor validity, prediction uncertainty or routing integrity exceeds configured thresholds.

# 13. Focused Literature Review and Design Influence

Only work that changes a concrete design decision is included. The project is not a reproduction of any single published architecture.

| Work | Relevant result | Project decision |
| --- | --- | --- |
| pi0 / openpi [1][2] | Specialized flow-matching action expert and action chunks for continuous robot control. | Adopt flow matching/action chunks for Motor Cortex; remove VLM inputs from the motor model. |
| pi0.5 / pi0.7 [3] | Hierarchical conditioning, action-expert separation and world/subgoal structure. | Keep Executive, predictive world representation and action model as distinct roles. |
| TD-MPC2 [4] | Planning by optimizing actions through a learned latent dynamics model. | Use candidate generation + learned prediction + deterministic scoring/receding horizon. |
| DreamerV3 [5] | Behavior improvement through imagined futures in a learned world model. | Treat prediction as a decision input rather than only an auxiliary loss. |
| I-JEPA [6] | Predict semantic target embeddings rather than reconstructing pixels. | Use JEPA objective for compact perception with limited parameters. |
| V-JEPA 2 [7][8] | Action-free video pretraining followed by action-conditioned robot training; planning in latent space. | Merge sensor representation and world modeling around a shared JEPA state + action-conditioned predictor. |
| Open X-Embodiment [9] | 1M+ trajectories across 22 embodiments; common gripper-frame action representation; positive transfer. | Use multi-robot data for general perception/world/task knowledge; normalize actions across embodiments. |
| DROID [10] | 76k demonstrations / ~350 h across 564 scenes and 86 tasks. | Use for real-world JEPA/world/executive generalization and language grounding. |
| BridgeData V2 [11] | 60,096 trajectories, 24 environments, 13 skills, language labels. | Use as additional diverse real manipulation pretraining. |
| Multi-Scale Embodied Memory [12] | Dense recent memory plus compressed long-term semantic memory. | Use working/short/long-term external stores controlled by one learned memory model. |
| ArmnetBench SO-101 [13] | 2,499 SO-101 episodes with success/failure/suboptimal labels. | Use embodiment-specific real data and negative/failure examples for critic/world/recovery training. |
| Project IRA SO-101 [14] | 930 SO-101 teleop episodes with 93 prompt variants. | Use for language-task grounding and SO-101 specialization while controlling environment bias. |
| NVIDIA SO-101 Isaac workflow [15] | Current SO-101 sim-to-real reference implementation on Isaac. | Reference only for SO-101 simulator/replay conventions; no custom paid Isaac data generation in baseline. |
| NVIDIA Transformer Engine / NVFP4 [16][17] | Blackwell supports NVFP4 mixed-precision with hierarchical block scaling. | Keep quantization interfaces modular; NVFP4 is deferred until Blackwell hardware is available at no required project cost. |

# 14. Current Development Baseline

## 14.1 Values fixed for V1 implementation

Primary embodiment: single-arm SO-101.

Camera baseline: 1-2 RGB cameras; add wrist view if available and throughput permits.

Common latent bus: 512D.

JEPA world state: approximately 32 tokens.

Motor control: 30 Hz initial; target 30-50 Hz.

Action horizon H=16; candidate count K=4; initial flow steps 8-10.

Motor commands: relative joint-position targets.

Motor Cortex input: q, qdot, previous command plus optional reliable actuator telemetry.

World prediction in latent space; precise body prediction remains numeric.

Memory is external; local Language/Speech retained.

Paid cloud budget = $0 for the baseline project. All mandatory training/integration must be achievable using the RTX 3050 4 GB laptop for tiny tests and RTX 3060 OC 12 GB for specialist training/integration.

## 14.2 Values to benchmark before locking

| Value | Baseline | Benchmark question |
| --- | --- | --- |
| JEPA image resolution | start 128-224 px crops | What is the minimum resolution that preserves object/pose grounding? |
| JEPA tokens | 32 | Does 16 lose grounding? Does 48/64 materially improve prediction? |
| Motor H | 16 | Does longer horizon improve planning enough to justify compute? |
| Candidate K | 4 | What K meets safety/goal accuracy within latency? |
| World predictor rate | 5-15 Hz | How often is environment prediction actually needed? |
| Executive context/vocab | task-dependent | Minimum size that supports local language and memory without wasting capacity. |
| RTX 3060 training recipe | microbatch 1 + accumulation/checkpointing + FP16 | Can each full specialist fit and train at useful throughput? |
| Local quantization exceptions | TBD by evaluation | Which modules/heads must remain FP16/FP32 if INT8/4-bit is attempted? |

# 15. Repository, Code and Data Folder Structure

The repository is organized so that model code, reusable neural-network building blocks, public-dataset adapters, training, evaluation and deployment remain separable. Raw data is immutable. Cleaned/normalized data is derived from raw data and can always be regenerated from pinned dataset revisions, manifests and preprocessing code.

## 15.1 Top-level repository layout

robot-brain/
|-- README.md
|-- pyproject.toml
|-- requirements/
|   |-- base.txt
|   |-- laptop_3050.txt
|   `-- train_3060.txt
|-- configs/
|   |-- models/
|   |-- training/
|   |-- datasets/
|   |   |-- registry/
|   |   `-- cleaning_profiles/
|   `-- deployment/
|-- src/
|   |-- nn/
|   |-- models/
|   |-- bus/
|   |-- data/
|   |-- replay/
|   |-- control/
|   |-- training/
|   |-- evaluation/
|   `-- deployment/
|-- scripts/
|-- tests/
|-- learn_lab/
|-- data/
|-- artifacts/
|-- notebooks/
|-- docs/
`-- tools/

| Folder | Purpose | Rule |
| --- | --- | --- |
| configs/ | YAML/TOML model, dataset, training and deployment configurations. | No hard-coded experiment settings inside model classes. |
| src/nn/ | Reusable low-level neural components: norms, attention, MLPs, positional encodings, transformer blocks. | Models import these blocks instead of duplicating implementations. |
| src/models/ | The eight project modules plus tiny test variants. | One package per model; architecture code stays separate from trainer code. |
| src/data/ | Dataset registry, source-specific adapters, schemas, cleaning, normalization, sampling, sharding and dataloaders. | Every transformation must be reproducible from raw data. |
| src/replay/ | Optional replay/inspection adapters for public Isaac/robot datasets. | No paid large-scale simulation generation is required. |
| src/control/ | IK, hard safety limits, MPC scoring and servo interfaces. | Deterministic control/safety is not hidden inside neural models. |
| src/training/ | Losses, optimizers, schedulers, distributed/mixed-precision helpers and trainer loops. | Shared infrastructure; model-specific loss adapters are thin. |
| src/evaluation/ | Offline metrics, rollout evaluators, calibration and regression tests. | A checkpoint is not promoted without repeatable metrics. |
| learn_lab/ | Small educational reimplementations written for understanding. | Do not import production model classes; rebuild concepts manually here. |
| artifacts/ | Checkpoints, logs, exports, quantization metadata and benchmark reports. | Never mix artifacts with source datasets. |

## 15.2 Recommended source-code package structure

src/
|-- nn/
|   |-- linear.py
|   |-- rmsnorm.py
|   |-- rope.py
|   |-- attention.py
|   |-- swiglu.py
|   |-- transformer_block.py
|   |-- embeddings.py
|   `-- masking.py
|
|-- models/
|   |-- jepa_encoder/
|   |   |-- model.py
|   |   |-- target_encoder.py
|   |   |-- predictor.py
|   |   `-- heads.py
|   |-- jepa_world/
|   |   |-- model.py
|   |   |-- action_adapter.py
|   |   `-- heads.py
|   |-- motor_cortex/
|   |   |-- model.py
|   |   |-- flow.py
|   |   `-- action_head.py
|   |-- body_dynamics/
|   |   |-- model.py
|   |   `-- uncertainty_head.py
|   |-- executive/
|   |   |-- model.py
|   |   |-- stage_jepa.py
|   |   `-- structured_head.py
|   |-- language_speech/
|   |   |-- audio_frontend.py
|   |   |-- conformer.py
|   |   |-- semantic_transformer.py
|   |   `-- intent_head.py
|   |-- memory/
|   |   |-- controller.py
|   |   |-- retriever.py
|   |   `-- compressor.py
|   `-- orchestrator/
|       |-- router.py
|       `-- schema_guard.py
|
|-- bus/
|   |-- schemas.py
|   |-- message.py
|   |-- adapters.py
|   `-- validation.py
|
|-- data/
|   |-- canonical_schema.py
|   |-- manifests.py
|   |-- adapters/
|   |-- converters/
|   |-- cleaning/
|   |-- normalization/
|   |-- splits.py
|   |-- sampling/
|   `-- dataloaders/
|
|-- training/
|   |-- trainer.py
|   |-- losses/
|   |-- optimizers.py
|   |-- schedulers.py
|   |-- precision.py
|   `-- checkpointing.py
|
|-- control/
|   |-- ik.py
|   |-- mpc.py
|   |-- safety.py
|   `-- so101_interface.py
|
`-- evaluation/
    |-- metrics.py
    |-- rollout_eval.py
    |-- sim2real.py
    `-- regression.py

## 15.3 Data directory: raw, cleaned and training-ready

data/
|-- raw/                         # immutable exact sources
|   |-- public_sim/
|   |   |-- so101_11task_3300/
|   |   |-- so101_ma_700/
|   |   |-- nvidia_singlearm/
|   |   |-- nvidia_augmented/
|   |   |-- mt_libero/
|   |   |-- dynasoftgrasp/
|   |   |-- assemblebench/
|   |   |-- assembly_joint/
|   |   |-- piper_apple_1m/
|   |   `-- ur5e_sfp/
|   |-- public_real/
|   |   |-- armnetbench_so101/
|   |   |-- project_ira_so101/
|   |   |-- open_x_embodiment/
|   |   |-- droid/
|   |   `-- bridgedata_v2/
|   `-- own_real/
|       |-- teleop/
|       |-- autonomous/
|       `-- calibration/
|
|-- cleaned/                     # validated canonical episodes
|   |-- public_sim/
|   |-- public_real/
|   `-- own_real/
|
|-- normalized/                  # units/actions/frames normalized
|   |-- native_actions/
|   |-- universal_ee_actions/
|   |-- jepa_sequences/
|   |-- world_sequences/
|   |-- language_tasks/
|   `-- failure_recovery/
|
|-- splits/                      # frozen split manifests
|-- shards/                      # model-specific packed tensors
|   |-- jepa_encoder/
|   |-- world/
|   |-- motor/
|   |-- body/
|   |-- executive/
|   |-- language/
|   |-- memory/
|   `-- orchestrator/
|
|-- manifests/
|   |-- dataset_registry.csv
|   |-- licenses.csv
|   |-- source_revisions.csv
|   |-- episodes.parquet
|   `-- preprocessing_runs.jsonl
`-- cache/                       # disposable local cache

| Data tier | Contents | Can be modified? | Examples |
| --- | --- | --- | --- |
| raw/ | Exact downloaded/generated source plus source metadata/checksums. | No. Append new versions only. | Original public_sim, public_real and own_real files plus source metadata/checksums. |
| cleaned/ | Corrupt samples removed; timestamps aligned; schema validated. | Regenerated, not hand-edited. | Canonical episodes with masks for missing sensors. |
| normalized/ | Units, coordinate frames and action representation standardized. | Regenerated from cleaned. | Radians, SI units, gripper-frame delta pose, SO-101 native joint deltas. |
| splits/ | Episode IDs for train/validation/test. | Versioned and frozen per experiment family. | No episode leakage across splits. |
| shards/ | Model-specific packed tensors for high-throughput training. | Disposable/regenerable. | WebDataset/Tar/Parquet/memmap-style shards. |
| cache/ | Temporary decoded images, indexes and downloaded subsets. | Yes. | Can be deleted at any time. |
| manifests/ | Dataset registry, revisions, licenses, unit/action semantics, preprocessing history. | Versioned; append new revisions. | dataset_registry.csv, source_revisions.csv, licenses.csv |

## 15.4 Mandatory data handling rules

Raw data is write-once. Cleaning bugs are fixed in code and the cleaned tier is regenerated; never repair raw episodes manually.

Every episode receives a stable episode_id, source_dataset, source_version, robot_id, license, success/failure label, task text, timestamps and quality flags.

Train/validation/test splitting is done by episode/scene/task where possible, never by individual adjacent frames, to prevent temporal leakage.

Images/video are referenced by timestamp/frame index; actuator and IMU streams keep their original timestamps before resampling.

Store both native robot action and normalized task-space action when available. The native SO-101 action remains the authoritative target for Motor Cortex/Body Dynamics specialization.

All normalization statistics are saved with the checkpoint. Never recompute test-set statistics during evaluation.

Dataset manifests include license, exact source revision, sim/real domain, robot embodiment, action semantics, camera schema and provenance before a source is admitted to training.

Large public datasets may remain on external storage; deterministic subset selectors and source-revision pins must make every experiment reproducible without copying all raw files into the repository.

## 15.5 Artifacts, checkpoints and experiment naming

artifacts/
|-- runs/
|   `-- <model>/<YYYYMMDD_name>/
|       |-- config.yaml
|       |-- git_commit.txt
|       |-- dataset_manifest.json
|       |-- metrics.jsonl
|       |-- logs/
|       `-- checkpoints/
|           |-- step_00010000.pt
|           |-- best.pt
|           `-- last.pt
|-- pretrained/
|-- integrated/
|-- quantized/
|   |-- scales/
|   |-- calibration/
|   `-- exported_models/
`-- reports/

Checkpoint rule: a checkpoint is identified by code commit + full config + dataset manifest + normalization state + precision mode. The weight file alone is not a reproducible model.

# 16. Implementation Learning Guide: From Basic Programming to Building the Models

This section is part of the implementation specification. Codex may generate production code, but every important neural component must have a small readable implementation and test so the architecture can be understood and later reimplemented without relying on generated code.

## 16.1 Minimum mental model

| Term | Meaning in this project |
| --- | --- |
| Tensor | A multidimensional numeric array. Example motor state batch: [batch, time, features]. |
| Shape | The size of each tensor dimension. Shape errors are among the most common implementation bugs. |
| Parameter | A learned tensor updated by the optimizer, e.g. Linear-layer weights. 1.024B means about 1.024 billion learned scalar parameters. |
| Activation | Intermediate values produced by a layer for one batch. Activations, not only parameters, consume VRAM during training. |
| Token | One element in a sequence processed by a Transformer. It may represent a word, image patch, world latent, sensor sample or action timestep. |
| Embedding | A learned/projection vector that turns raw IDs or numeric inputs into d_model-dimensional tokens. |
| d_model / hidden size | Width of each Transformer token vector, e.g. 768. Larger width strongly increases parameter and compute cost. |
| Sequence length | Number of tokens processed together. Attention memory/compute generally grows rapidly with sequence length. |
| Batch | Independent samples processed in parallel. On 12 GB VRAM, microbatch may be 1 with gradient accumulation. |
| Forward pass | Inputs -> model -> predictions/loss ingredients. |
| Loss | Scalar objective measuring error; training minimizes it. |
| Backward pass | Automatic differentiation computes gradients of loss with respect to parameters. |
| Gradient | Direction/magnitude telling the optimizer how each parameter should change. |
| Optimizer | Algorithm such as AdamW that applies gradients to weights. |
| Learning rate | Step size used by optimizer; usually scheduled over training. |
| Step | One optimizer update. With accumulation, several microbatches may produce one optimizer step. |
| Epoch | One pass over a finite dataset; less useful than step counts for very large mixed datasets. |
| Checkpoint | Saved model + optimizer/training state used to resume or evaluate. |
| Inference | Using a trained model without backpropagation. |
| Mixed precision | Using lower-precision arithmetic for speed/VRAM while retaining selected operations in higher precision. |
| Quantization | Representing trained weights/activations with fewer bits (for example INT8 or weight-only 4-bit) to reduce memory/inference cost. NVFP4 is an optional future Blackwell path, not the baseline. |

## 16.2 Transformer block anatomy

x = input tokens                       # [B, T, d_model]

x = x + Attention(RMSNorm(x))        # communicate between tokens
x = x + SwiGLU_FFN(RMSNorm(x))       # transform each token independently

return x

| Component | Function | Why used here |
| --- | --- | --- |
| Linear projection | y = xW + b; changes feature width and learns mixtures of input features. | Sensor adapters, Q/K/V projections, MLPs, prediction heads. |
| RMSNorm | Normalizes token magnitude without subtracting mean. | Stabilizes deep Transformers with low overhead. |
| Q/K/V attention | Queries choose which keys to read; values carry the information. | Lets tokens exchange context across time, sensors, actions or language. |
| Multi-head attention | Runs several attention subspaces in parallel. | Different heads can represent different relations/time scales. |
| GQA | Many query heads share fewer key/value heads. | Reduces KV memory and inference cost. |
| RoPE | Rotates Q/K vectors based on position. | Adds sequence/time order without a large learned positional table. |
| SwiGLU FFN | Gated feed-forward nonlinear transformation applied per token. | Provides most per-token feature transformation capacity. |
| Residual connection | Adds layer input to layer output. | Preserves information and makes deep optimization stable. |
| Causal mask | Blocks attention to future tokens. | Required in autoregressive Executive/text decoding; not automatically required in JEPA encoders. |

## 16.3 What to implement manually first

Linear layer usage and tensor shapes; do not initially implement matrix multiplication kernels yourself.

RMSNorm from its formula, then compare output to a trusted framework implementation.

Single-head scaled dot-product attention; visualize its attention matrix on a tiny sequence.

Multi-head attention, then GQA.

RoPE and attention masks.

SwiGLU feed-forward layer.

Pre-norm TransformerBlock with two residual connections.

A 2-4 layer tiny Transformer that overfits a toy sequence task.

Only then implement JEPA, flow matching, Conformer and the project-specific models.

## 16.4 Model-by-model layer map

### 16.4.1 Multimodal JEPA Encoder

| Stage/layer | Input -> output | Function |
| --- | --- | --- |
| Image patch embedding | RGB [B,C,H,W] -> patch tokens [B,N,d] | Conv/linear projection converts image patches into Transformer tokens. |
| Sensor adapters | q/qdot/IMU/etc. -> sensor tokens | Small MLPs/Linear layers put numeric sensor streams in the same hidden space. |
| Type/time/view embeddings | tokens + metadata -> tokens | Tell the model which camera/sensor and time each token came from. |
| Context Transformer | visible tokens -> contextual tokens | Learns spatial/temporal relationships without reconstructing pixels. |
| Fusion blocks | camera + IMU + proprioception -> fused tokens | Allows information exchange between modalities. |
| Resampler | many tokens -> about 32 world tokens | Caps downstream sequence length and parameter/compute use. |
| Target encoder (EMA) | future/held-out observation -> target latent | Provides stable JEPA training targets; normally updated by exponential moving average, not direct gradient descent. |
| JEPA predictor | context latent + mask/time -> predicted target latent | Learns to predict missing/future semantic representation. |
| Optional supervised heads | world tokens -> class/color/pose/etc. | Adds explicit grounding required by tasks such as RED + BALL. |

### 16.4.2 JEPA World Predictor

| Stage/layer | Function |
| --- | --- |
| World-state adapter | Projects current JEPA world tokens into predictor hidden width. |
| Action/subgoal adapter | Embeds candidate action trajectory or physical subgoal. |
| Action-conditioned Transformer | Models how current world state changes under a candidate action. |
| Future latent head | Predicts z_hat at one or more future horizons. |
| Event heads | Predict grasp/contact/collision/object-motion probabilities when labels exist. |
| Uncertainty head/calibration | Estimates how reliable the prediction is for planning. |

### 16.4.3 Motor Cortex - flow-matching action model

| Stage/layer | Function |
| --- | --- |
| State adapter | Projects q, qdot, previous command and optional actuator telemetry to state tokens. |
| Goal adapter | Projects q_goal, qdot_goal, duration and constraints to goal tokens. |
| Noisy action tokens | During training, represent interpolated/noisy H x 6 action trajectory. |
| Flow-time embedding | Represents scalar flow time t; conditions the network on the current denoising/transport point. |
| AdaRMSNorm/equivalent conditioning | Modulates Transformer normalization using flow-time information. |
| Motor Transformer | Combines current body state, goal and current noisy action trajectory. |
| Vector-field head | Predicts the direction/velocity that moves noisy actions toward the demonstrated action distribution. |
| Flow sampler | At inference starts from random noise and integrates several steps to produce K candidate action chunks. |

### 16.4.4 Body Dynamics

| Stage/layer | Function |
| --- | --- |
| State embedding | Encodes current SO-101 q/qdot/load. |
| Action sequence embedding | Encodes one H-step candidate action chunk. |
| Temporal Transformer | Predicts how actuator state evolves through the candidate trajectory. |
| Mean head | Predicts future numeric q/qdot/etc. |
| Log-variance / scale head | Predicts uncertainty while enforcing positive variance through a stable transform such as softplus/clamping. |
| Residual real-dynamics adapter | Optional fine-tuned correction f_real = f_sim + f_residual. |

### 16.4.5 Executive Brain

| Stage/layer | Function |
| --- | --- |
| Text/token embedding | Embeds task-language tokens and discrete control symbols. |
| Bus adapters | Projects 512D world/memory/prediction tokens into Executive hidden width. |
| Decoder Transformer | Autoregressively reasons over task goal, observations, memory and predictions. |
| Causal mask | Prevents generated reasoning/output tokens from reading future output positions. |
| Stage-JEPA predictor | Predicts a useful next-task-stage latent; auxiliary to the main decoder. |
| Structured output head/grammar | Produces validated subgoal fields rather than unrestricted prose for control. |

### 16.4.6 Language + Speech

| Stage/layer | Function |
| --- | --- |
| Audio frontend | Waveform -> log-Mel/filterbank features. |
| Conformer blocks | Combine self-attention for long context with convolution for local acoustic patterns. |
| Text/semantic Transformer | Maps transcript/text context into semantic task representation. |
| Intent/entity heads | Predict intent, object class, attributes, relations and constraints. |
| Optional text decoder | Generates local language responses if needed. |

### 16.4.7 Memory Controller

| Stage/layer | Function |
| --- | --- |
| Query encoder | Turns Executive request/current state into retrieval query tokens. |
| Retriever/scorer | Scores external memory entries by relevance, recency and confidence. |
| Write gate | Decides ignore/store/update/summarize. |
| Compressor | Converts long/repetitive episodes into compact semantic records. |
| Staleness/conflict logic | Flags old or contradictory evidence instead of treating all memories equally. |

### 16.4.8 Orchestrator

| Component | Function |
| --- | --- |
| Schema guard - deterministic | Validates message type, shape, units, timestamp and required fields. |
| Priority/timeout logic - deterministic | Rejects stale messages and prevents slow modules from blocking safety-critical control. |
| Small router network | Chooses module destination when routing is ambiguous or learned prioritization helps. |
| Correlation IDs | Connect requests and replies, e.g. one candidate-action request to its body/world predictions. |

## 16.5 Losses you must understand

| Loss/objective | Used by | Meaning |
| --- | --- | --- |
| MSE / L2 | Toy models, numeric predictions | Squared prediction error; simple but sensitive to outliers. |
| Huber | Body state/pose regression | Quadratic near zero and linear for large errors; robust numeric regression. |
| Cross-entropy | Language, class/event heads | Probability loss for discrete classes/tokens. |
| JEPA latent loss | JEPA Encoder/World Predictor | Makes predicted latent match target-encoder latent without reconstructing pixels. |
| Flow-matching loss | Motor Cortex | Matches predicted action-space vector field to the target transport/vector field. |
| Gaussian NLL | Body uncertainty | Rewards accurate mean prediction and calibrated predictive variance. |
| Auxiliary goal/smoothness penalties | Motor training/evaluation | Encourage goal completion and smooth trajectories; do not replace the main demonstrated-action objective. |

## 16.6 Training-loop anatomy

model.train()
for batch in dataloader:
    batch = move_to_device(batch)

    with autocast_if_enabled():
        outputs = model(batch)
        loss = compute_loss(outputs, batch)
        loss = loss / grad_accum_steps

    scaler_or_loss.backward()

    if accumulation_boundary:
        clip_gradients_if_configured()
        optimizer.step()
        scheduler.step()
        optimizer.zero_grad(set_to_none=True)

    log(loss, learning_rate, metrics)
    periodically_save_checkpoint()

You should be able to explain every line above before moving from tiny laptop models to full-size training. PyTorch automatic differentiation computes the derivatives; the project code is responsible for constructing the correct computation graph, loss, masks, precision policy and optimizer step.

## 16.7 Educational implementation workflow with Codex

1. For every new neural concept, first ask Codex to describe the tensor shapes and equations before asking for production code.

2. Create a minimal version in learn_lab/ using tiny dimensions and synthetic data. Keep it under roughly 200 lines when possible.

3. Write unit tests that compare shapes, finite outputs, gradients and known invariants. A neural layer that runs is not necessarily correct.

4. Make the tiny model intentionally overfit 8-64 examples. Failure to overfit usually indicates a code/data/loss bug.

5. Only after the toy implementation is understood, use the shared src/nn/ production implementation in the full model.

6. When Codex generates a model class, require a short architecture comment giving every input/output shape and parameter-producing submodule.

7. Do not accept unexplained performance optimizations. First keep a readable reference implementation; add FlashAttention, compilation, checkpointing and quantization behind tested interfaces.

8. Maintain docs/terminology.md and docs/tensor_shapes.md as living references discovered during implementation.

## 16.8 Learning milestones tied to project hardware

| Milestone | Hardware | What you should personally understand before advancing |
| --- | --- | --- |
| A. Neural basics | CPU / RTX 3050 | Tensor shapes, Linear, loss, backward, AdamW, checkpoint; overfit tiny MLP. |
| B. Transformer basics | RTX 3050 | RMSNorm, attention, RoPE, SwiGLU, residuals, masks; build tiny Transformer. |
| C. Project objectives | RTX 3050 | JEPA target/context encoder, flow-matching objective, uncertainty/NLL; run toy versions. |
| D. Tiny eight-model bus | RTX 3050 | Typed interfaces, adapters, mock messages, save/load, deterministic tests. |
| E. Full specialists | RTX 3060 12 GB | Gradient accumulation, activation checkpointing, mixed precision, dataloaders, metrics. |
| F. Dataset engineering | CPU / RTX 3050 / RTX 3060 | How heterogeneous public sim/real data becomes raw -> cleaned -> normalized -> model-specific shards with pinned provenance. |
| G. Integrated robot brain | RTX 3060 + real SO-101 | Frozen/unfrozen modules, offline replay, real teleop adaptation, reference metrics and optional local quantization. |

## 16.9 Functions that should remain framework/library code

Learning the architecture does not require reimplementing CUDA kernels, autograd or optimizers from mathematical primitives. Use PyTorch/approved libraries for the following while understanding their interfaces and equations:

Tensor kernels, matrix multiplication and CUDA memory management.

Automatic differentiation/backpropagation engine.

AdamW and standard learning-rate schedulers.

Production scaled-dot-product/FlashAttention kernels after the reference attention implementation is tested.

Image/video codecs, distributed collectives and filesystem primitives.

Hardware-specific low-precision kernels (including NVFP4) remain framework/library work and are optional future deployment paths, not required project code.

Isaac Sim/Isaac Lab replay and physics primitives are framework code; public datasets may be inspected/replayed without making simulation generation a mandatory project stage.

## 16.10 Definition of "I understand this model"

Before considering a module learned rather than merely generated, you should be able to answer these questions without reading the source:

What is every input tensor, its units and its shape?

What does one token represent?

Which layers contain learned parameters and approximately where most parameters are spent?

What attention mask is used and why?

What loss trains the model and what target creates that loss?

How does information leave the model and which downstream module consumes it?

What failure would appear if this model were removed or badly trained?

Which parts can be quantized locally without degrading physical accuracy, and which motor/state/uncertainty outputs must remain FP16/FP32?

How can a tiny version be made to overfit a synthetic dataset?

Which metric proves that a new checkpoint is actually better?

# 17. References

[1] Black et al. (2024). pi0: A Vision-Language-Action Flow Model for General Robot Control. https://arxiv.org/abs/2410.24164

[2] Physical Intelligence. openpi repository. https://github.com/Physical-Intelligence/openpi

[3] Physical Intelligence / Black et al. pi0.5 and pi0.7 project material. https://arxiv.org/abs/2504.16054 ; https://www.pi.website/blog/pi07

[4] Hansen, Su, Wang (2023). TD-MPC2: Scalable, Robust World Models for Continuous Control. https://arxiv.org/abs/2310.16828

[5] Hafner et al. (2023). Mastering Diverse Domains through World Models (DreamerV3). https://arxiv.org/abs/2301.04104

[6] Assran et al. (2023). Self-Supervised Learning from Images with a Joint-Embedding Predictive Architecture (I-JEPA). https://arxiv.org/abs/2301.08243

[7] Meta AI (2025). Introducing the V-JEPA 2 world model and benchmarks for physical reasoning. https://ai.meta.com/blog/v-jepa-2-world-model-benchmarks/

[8] Meta AI (2025). V-JEPA 2: Self-Supervised Video Models Enable Understanding, Prediction and Planning. https://ai.meta.com/research/publications/v-jepa-2-self-supervised-video-models-enable-understanding-prediction-and-planning/

[9] Open X-Embodiment Collaboration (2023). Open X-Embodiment: Robotic Learning Datasets and RT-X Models. https://robotics-transformer-x.github.io/

[10] Khazatsky et al. DROID: A Large-Scale In-the-Wild Robot Manipulation Dataset. https://droid-dataset.github.io/

[11] Walke et al. BridgeData V2: A Dataset for Robot Learning at Scale. https://rail-berkeley.github.io/bridgedata/

[12] Physical Intelligence (2026). Multi-Scale Embodied Memory. https://www.pi.website/research/memory

[13] ArmnetBench v0.1 - LeRobot SO-101 dataset. https://huggingface.co/datasets/armnet/armnetbench_v01_lerobot_so101

[14] Project IRA SO-101 Multi-Task Dataset - 930 episodes. https://huggingface.co/datasets/Project-IRA/TPSoSe2026_Dataset_Full_Merged_Final_LeRobot_SO101_V1

[15] NVIDIA / Isaac Sim. Sim-to-Real SO-101 Workshop. https://github.com/isaac-sim/Sim-to-Real-SO-101-Workshop

[16] NVIDIA Transformer Engine - Getting Started / precision support. https://docs.nvidia.com/deeplearning/transformer-engine/user-guide/getting_started/index.html

[17] NVIDIA Transformer Engine - FP8 and NVFP4 training primer. https://docs.nvidia.com/deeplearning/transformer-engine/user-guide/examples/fp8_primer.html

[18] Hugging Face LeRobot documentation / SO-101. https://huggingface.co/docs/lerobot/main/en/assemble_so101

[19] User-provided Public Isaac Sim / Isaac Lab Robot-Arm Manipulation Datasets inventory (checked 28 Aug 2026).

[20] NVIDIA PhysicalAI Robotics Manipulation SingleArm. https://huggingface.co/datasets/nvidia/PhysicalAI-Robotics-Manipulation-SingleArm

[21] NVIDIA PhysicalAI Robotics Manipulation Augmented. https://huggingface.co/datasets/nvidia/PhysicalAI-Robotics-Manipulation-Augmented

[22] China-SAE Robotics MT-Libero-Dataset. https://huggingface.co/datasets/china-sae-robotics/MT-Libero-Dataset

[23] DynaSoftGrasp M1 Soft Mouse. https://huggingface.co/datasets/KhalilGao/dynasoftgrasp-m1-softmouse-pick

[24] SoftVTBench. https://github.com/TuojingAI/SoftVTBench

[25] AssembleBench. https://huggingface.co/datasets/hud-evals/AssembleBench

[26] Assembly Joint. https://huggingface.co/datasets/lukasskellijs/assembly_joint

[27] Piper Apple Picking 1M. https://huggingface.co/datasets/Faless/piper-apple-picking-1m

[28] SO101 MA MultiTask 700. https://huggingface.co/datasets/CoRL2026-CSI/SO101-MA-MultiTask-700epi_10fps

# 18. Revision Note - v1.3

v1.3 changes the compute and data scope. Paid GB100 rental and mandatory custom Isaac data generation are removed. Training is now based on diverse public synthetic + real multi-embodiment datasets, followed by public and self-collected SO-101 specialization on existing local hardware. The data section now includes a pinned dataset registry, explicit sim/real/embodiment tags, model-specific dataset routing, per-model cleaning/sharding rules, sampling weights, and an updated repository layout separating public_sim, public_real and own_real data. The 1.024B allocation is also revised toward shared JEPA perception/world capacity: 280M JEPA Encoder, 160M World Predictor, 330M Executive, 128M Motor, 32M Body, 80M Language/Speech, 12M Memory and 2M Orchestrator.